Envision - Assignment 2

FIRST OF ALL. I want u guys to set up Cmsis device headers.

Avoid manually hardcoding peripheral addresses and dereferencing pointers, instead, use CMSIS device headers to access registers.

https://drive.google.com/drive/folders/1ciHfyqHaYyizKpiHeYUxyyH5MEOkAbeA?usp=sharing

Download these and Add the paths to these folders to ur project

Right click on ur project → Properties → C/C++ General →Paths and Symbols→Add.. → File system—> then add the paths to these 2 folders

1. CMSIS libraries\Device\ST\STM32F4xx\Include

2. CMSIS libraries\Include

Then finally u can do #include “stm32f446xx.h” in ur files and use the predefined macros and dont have to keep looking for addresses.





















Question 1

Design a bare-metal STM32 project using CMSIS (register-level macros only) to read an analog voltage from a potentiometer and convert it into a digital value using the ADC.

The system should:

- Continuously sample the analog input from the potentiometer

- Convert it into a digital value using the ADC

- Compute the corresponding voltage

- Transmit the readings over UART

The UART terminal should update in real time as the potentiometer knob is rotated.

- Do not hardcode peripheral addresses or manually dereference pointers

- All register interactions must be done using CMSIS-defined macros/structures

Components Required

- Potentiometer

- Breadboard

- Connecting wires

(Collect from Analog Lab, ECE Dept, or coordinate among yourselves)

Your README should include:

- What is an ADC?

- What is resolution in ADC?

- Identify the type of ADC used in STM32 (SAR ADC)

- Explain its working mechanism briefly

- List the registers used (ADC, GPIO, UART, RCC, etc.)

- Provide a short description of each

- Include a final implementation video showing:

  - Hardware setup

  - UART output changing with potentiometer rotation

Question 2

Design a bare-metal STM32 project using CMSIS (register-level macros only) to generate an analog output voltage using the DAC.

The system should:

- Generate a digital value in software

- Convert it into an analog voltage using the DAC

- Output the voltage through the DAC pin

- Vary the output dynamically (e.g., ramp, sine wave, or based on user-defined values)

Optionally, verify the output using a multimeter or oscilloscope.

Create a new folder inside your main repository: Assignment_X (no need for new repository)

- Explain how digital values are converted into analog voltage

- Mention the type of DAC used in STM32 (typically resistor-string DAC)

- List the registers used (DAC, GPIO, RCC, etc.)

- Include a final implementation video showing:

  - Hardware setup

  - Output behavior (changing voltage / waveform)

Preferable deadline : 
Question 1 - 16th April EOD

Question 2 -  try it after ur endsems